ELECTROMYOGRPAHY

What is electromyography?

- Electromyography (EMG) is a technique used to measure the electrical activity produced by skeletal muscles.

- When your brain sends a signal to contract a muscle, motor neurons activate groups of muscle fibres called motor units.

- The activation of muscle fibres generates tiny electrical signals known as muscle action potentials.

- Surface EMG records these signals using electrodes placed on the skin above the muscle.

- The strength of the EMG signal generally increases as more motor units are recruited and muscles produce greater force.

- EMG does not measure force directly; it measures the electrical activity associated with muscle activation.

- EMG signals are typically only a few microvolts to millivolts in amplitude and must be amplified and processed before analysis.

- Signal-processing techniques such as filtering, rectification, and smoothing are often used to extract useful information from raw EMG data.

Why is EMG useful?

- EMG is widely used in biomechanics, rehabilitation, sports science, prosthetics, and human-computer interaction.

- It provides insight into how the nervous system controls movement.

- EMG can be used as an intuitive control signal for assistive devices, virtual reality systems, and interactive games.